TriChIMES: A GPU Accelerated Machine Learning Interatomic Potential with Triton Kernels

Philip Alcorn1,2 and Dylan M. Anstine1

1 Department of Chemical Engineering and Materials Science, Michigan State University, East Lansing, Michigan, 48824, USA

2 Department of Electrical Engineering and Computer Science, Tarleton State University, Stephenville, Texas, 76401, USA

Traditional electronic structure calculations, particularly Density Functional Theory (DFT), offer an accurate description of molecular and material systems but their computational demands limit the system sizes and/or number of systems that can be simulated. Machine learned interatomic potentials (MLIPs) have been developed to overcome this unfavorable scaling, whereby the potential energy of a surface is approximated by training a data-driven model to reproduce the DFT reference data. In this work, we report a GPU optimized implementation of the Chebyshev Interaction Model for Efficient Simulation (ChIMES) MLIP which can perform molecular simulations several orders of magnitude faster than the reference CPU-based version. Kernels for neighbor list building as well as N-body order energy and force contributions have been efficiently implemented in the single-program multiple data (SPDM) Triton programming model. Our SPMD implementation takes a fused-kernel approach, simultaneously calculating energy, forces, and parameter gradients for training, minimizing costly host-device data transfers and the overall memory footprint by avoiding the need for storing intermediate results. The kernels exploit a blocked calculation design, where energy and force contributions from a central atom and all its local neighbors are calculated across single warp partitions in parallel. The set of TriCHIMES kernels have been integrated into a collection of custom automatic gradient (autograd) functions that support efficient MLIP training within the PyTorch ecosystem. We have trained a 1st generation TriCHIMES model on the ANI-1x training dataset (5M structures), which shows competitive accuracy with MLIP model of similar computational cost.